# Building PID Controller using OP AMPS and Passing Time Varying Voltage Signals Through It

what to write

sabki equations

summing wale ki equations

saare parts ki values

uske baad sine wave + pics

4square wave + pics

saw + pics

uske baad trouble shooting

with 12 v supply clipping starts at output boltage = 24 volts trial and error

CIRCUIT SPECIFICS

INPUT VOLTAGE +12, -12, 0 GROUND

PROPORTIONAL CONTROL

Rin = 5k

Rf = 20k

Rout = 10k

gain = -Rf/Rin = 4 times

180 degree phase difference/flipped/amplified output

Sin amplified to 8v

square amplified to

saw amplified to

INTEGRAL CONTROL

Rin = 3K

Cf = 10k

Rf = 100nF

Rout = 10k

See maths in notebook

gain =

R Vout = 31830.98

R = 31,83.098 , input value of Vo to get in that range  for Vo = 10 (divided by Vo)

-----------------------------------------------

differential control

rf = 10/smthe

rf = 79577.47

rf = 80 k

vo = 10

- Sine Wave Test:

  - Frequency: 100 Hz

  - Amplitude: 2.0 V

  - DC Offset: 0.0 V

  - Function Type: Sine

- Square Wave Test:

  - Frequency: 50 Hz

  - Amplitude: 2.5 V

  - DC Offset: 0.0 V

  - Function Type: Square

- Sawtooth/Triangle Wave Test:

  - Frequency: 100 Hz

  - Amplitude: 2.0 V

  - DC Offset: 0.0 V

  - Function Type: Triangle

TROUBLESHOOTING

- Why use large resistance?

to limit current drawn from function generator, heat loss and all

- Output Voltage from Op Amp gets clipped
If Gain in inverting amplifier is very high such that output voltage is more than what power supply of amplifier can support, output wave form gets clipped.

Eg: initially took Proportional control with 9 V supply, input signal amplitude = 2V, gain = 10. Output signal = 20V was clipped at the top.

This problem was resolved using a 12 V supply, which can comfortably handle gain in given situation.

- Negative DC offset on output waveform of Integral Control

An Ideal integral controller would use only a capacitor in the feedback loop.

For non-ideal case, small error DC voltage is produced due to minor voltage difference between inverting and non-inverting inputs, Capacitor acts as open circuit for DC and gets charged. Thus, these small voltages don’t have a feedback loop and hence form an open loop inverting amplifier. Gain in open loop inverting amplifier is very large

(around200,000) hence these small voltages are amplified to noticeable amount in output which causes the negative DC offset

FIX: Attach a large resistor in parallel to the capacitor

This creates a feedback loop for the small DC voltage errors, creating a closed loop inverting amplifier for them, which reduces the current gain (gain is lower in closed loop compared to open loop Gain = -Rf/Rin). Resistor is large so that AC signal doesn’t flow back from it.

- Connecting the Derivative control caused the input signal to flatline.

At high frequency AC signals, the capacitive reactance (=1/2pifC) becomes very low, and the input capacitor behaves like a virtual short circuit essentially connecting the positive end of the function generator directly to the inverting input which acts as a virtual ground (stays close to 0). The negative terminal of function generator is connected to ground and the positive terminal got connected to a virtual ground (as explained above), so we essentially shorted our function generator which caused the signal it generated to collapse and caused the other controllers to show haphazard outputs.

FIX : attach a small resistance in series with the input capacitor.

- Jaggedness of surface – small errors being amplified by the differentiator

- fix how ? Increasing the input resistor reduces jaggedness

- cutoff voltage both integrator and diffrentiator

The reason your input oscilloscope flatlined and the other stages went crazy is that your Derivative (D) stage is acting as a short circuit and dragging down the entire function generator.

Because all three stages (P, I, and D) share the same main input node, a short circuit in one stage kills the signal for all of them.

Why the Derivative Stage Causes a Short Circuit

A textbook Derivative controller connects a capacitor directly from the input signal to Pin 2 of the op-amp.

- Capacitors act as a solid wall to DC, but they act like a piece of wire (a short circuit) to high frequencies and sudden voltage changes (like the sharp edges of square and triangle waves).

- Because Pin 2 is a "virtual ground" (0 V), the high-frequency parts of your input wave pass straight through the capacitor directly into ground. This literally short-circuits your function generator, which is why your input oscilloscope flatlined.

- Tinkercad's simulation engine panics when this happens, resulting in the jagged edges and square pulses you see on the other scopes.

How to Fix the D Stage (The "Practical Differentiator")

To fix this, you must add a resistor in series with the input capacitor to limit the current and stop it from shorting out the input node.

1.Disconnect the D Stage Temporarily:Isolate the problem.

Delete the wire connecting the main input node to your Derivative capacitor. Run the simulation. Your input wave, P stage, and I stage should instantly return to normal.

2.Add a Series Resistor to the Input:Limit the high-frequency current.

Place a 1 kΩ resistor directly in series with your D-stage input capacitor.

- The path should be: Main Input Node → 100 nF Capacitor → 1 kΩ Resistor → Pin 2 of the D Op-Amp.

- This stops the capacitor from acting like a 0-ohm wire during fast voltage changes, saving your input signal.

3.Verify the Feedback Resistor:Check the feedback loop.

Ensure you have a 10 kΩ resistor connected between Pin 2 and Pin 6 of the Derivative Op-Amp. Without this, the op-amp will run at infinite gain and saturate immediately into a square wave.

4.Check Pin 3:Keep it grounded.

Just like the other stages, make sure Pin 3 of the D Op-Amp is securely connected to your System Ground.

Once you add that 1 kΩ series resistor, the simulation will stabilize, the input wave will return, and the Derivative scope will show the proper mathematical derivative (e.g., a triangle wave input will turn into a square wave output).